Ain Shams University

Faculty of Engineering

Computer and Systems Engineering Department

CSE 412: Digital Verification

4th Year CSE

2nd Semester 2021/2022

Assignment 2

Submitted by:

Sarah Mohamed Ahmed Ahmed

Code: 1700593

Section: 2

Submitted to:

Dr. Ayman Wahba

Github link: https://github.com/Sarah-56/Digital-Verification/tree/main/latest

Table of Contents

Design:	3

Interface:	3

Counter Module:	3

Test bench:	5

Top module:	7

Assertion output:	8

Output:	9

# Design:

## Interface:

interface Counter_Interface #(

parameter COUNTER_SIZE = 4

)(

input bit clk

);

bit [1:0] ctrl, WHO;

bit INIT, LOSER, WINNER, GAMEOVER, rst_l;

bit [COUNTER_SIZE - 1:0] loadValue;

clocking cb @(posedge clk);

default input #0ns output #1ns;

output rst_l, ctrl, INIT, loadValue;

input WHO, LOSER, WINNER, GAMEOVER;

endclocking

modport dut(

output GAMEOVER, WHO, LOSER, WINNER,

input clk, rst_l, ctrl, INIT, loadValue

);

modport tb

(

clocking cb,

output rst_l

);

endinterface

## Counter Module:

module counter #(

parameter COUNTER_SIZE = 4          // number of bits in counter

)(

Counter_Interface.dut sig

);

/*********************************************************************

PARAMETERS

*********************************************************************/

parameter cycle = 2;         // clock cycle = 2 msec.

parameter whoValue = 2'b00;  //start value

parameter upOne = 2'b00;     //00 count up by 1

parameter upTwo = 2'b01;     //01 count up by 2

parameter downOne = 2'b10;   //10 count down by 1

parameter downTwo = 2'b11;   //11 count down by 2

/********************************************************************

REGISTERS & WIRES

**********************************************************************/

reg LOSER;

reg WINNER;

reg GAMEOVER;

reg [1:0] WHO;

reg [1:0] ctrl;

reg [COUNTER_SIZE - 1:0] m_counter;

reg [COUNTER_SIZE - 1:0] loser_count;

reg [COUNTER_SIZE - 1:0] win_count;

wire [COUNTER_SIZE - 1:0] loadValue;

/*********************************************************************

ALWAYS BLOCK

*********************************************************************/

always @(posedge sig.clk) begin

if (sig.INIT) begin

m_counter = sig.loadValue;

sig.WHO = whoValue;

loser_count = 0;

win_count = 0;

sig.LOSER = 0;

sig.WINNER = 0;

sig.GAMEOVER = 0;

end

else begin

/*************************

SYNCHRONOUS RESET

*************************/

if (sig.rst_l || sig.GAMEOVER) begin

m_counter <= 4'b0000;

sig.LOSER <= 0;

sig.WINNER <= 0;

sig.WHO <= 2'b00;

loser_count <= 0;

win_count <= 0;

sig.GAMEOVER <= 0;

end

/*************************

INITIALIZATION

*************************/

else if (sig.INIT) begin

m_counter <= sig.loadValue;

loser_count <= 0;

win_count <= 0;

sig.WHO <= 2'b00;

sig.WINNER <= 0;

sig.LOSER <= 0;

sig.GAMEOVER <= 0;

end

else begin

case (sig.ctrl)

upOne:  m_counter <= m_counter + 1;

upTwo:  m_counter <= m_counter + 2;

downOne:  m_counter <= m_counter - 1;

downTwo:  m_counter <= m_counter - 2;

endcase

/*************************************************************************

set LOSER signal to 1 for 1 clock cycle then clear it and increase

loser counter by 1 if counter reaches 0

*************************************************************************/

if(m_counter == 0) begin

sig.LOSER <= 1;

sig.WINNER <= 0;

loser_count = loser_count + 1;

end

/**********************************************************

set WINNER signal to 1 for 1 clock cycle then clear

it and increase

winner counter by 1 if counter reaches 15

***********************************************************/

else if(m_counter == 15) begin

sig.WINNER <= 1;

sig.LOSER <= 0;

win_count = win_count + 1;

end

else begin

sig.LOSER <= 0;

sig.WINNER <= 0;

end

// raise gameover signal if loser or winner counter reaches 15

if(loser_count == 15 || win_count == 15) begin

sig.GAMEOVER <= 1;

if(loser_count == 15) sig.WHO <= 2'b01;

else sig.WHO <= 2'b10;

end

end

end

end

endmodule

## Test bench:

program tb_counter(Counter_Interface.tb sig);

/********************************************************************************

PARAMETERS

********************************************************************************/

parameter cycle = 2;

parameter COUNTER_SIZE = 4;

/********************************************************************************

INITIAL BLOCK

********************************************************************************/

initial begin

sig.cb.loadValue <= 4'b0000;

sig.cb.ctrl <= 2'b00;

sig.cb.rst_l <= 0;

sig.cb.INIT <= 1;

for (int ctrl_c = 0; ctrl_c <= 3; ctrl_c = ctrl_c + 1) begin

for (int loadValue_c = 0; loadValue_c < 3; loadValue_c = loadValue_c + 1) begin

// sig.rst_l <= 1;

assertion_1: assert (sig.cb.WINNER == 0)

$display("WINNER = %d asserted correctly", sig.cb.WINNER);

else

$fatal("WINNER = %d not asserted correctly", sig.cb.WINNER);

sig.cb.ctrl <= ctrl_c;

if(loadValue_c == 2) sig.cb.loadValue <= {COUNTER_SIZE{1'b1}};

else sig.cb.loadValue <= loadValue_c;

sig.cb.INIT <= 0;

#2

sig.cb.rst_l <= 0;

#2

sig.cb.INIT <= 1;

#1

sig.cb.INIT <= 0;

#481

sig.cb.rst_l <= 1;

end

end

end

/****************************

Assign BLOCK

****************************/

assign WHO = sig.cb.WHO;

assign LOSER = sig.cb.LOSER;

assign WINNER = sig.cb.WINNER;

assign GAMEOVER = sig.cb.GAMEOVER;

/****************************

Properties

****************************/

property reset_signals;

@(sig.cb) disable iff(!($fell(sig.rst_l) )) (WHO ==0 || LOSER == 0 || GAMEOVER == 0 || WINNER ==0);

endproperty

property winner;

@(sig.cb)

if($fell(sig.rst_l)) ##[150:250] GAMEOVER == 1;

endproperty

/****************************

Asserions

****************************/

assert_winner: assert property(winner)$display("@ cycle [%0t] Assertion GameOver passed", $time / 2);

assert_reset_signals: assert property (reset_signals) $display("@ cycle [%0t] Assertion Reseting signals passed", $time / 2);

endprogram

## Top module:

module top (output bit clk);

initial clk = 1;

initial forever #1 clk = ~clk;

Counter_Interface iface(clk);

tb_counter t0(iface.tb);

counter G0(iface.dut);

/******************************

DUMP VARIABLES

******************************/

initial begin

$dumpfile("wave.vcd");

$dumpvars;

end

endmodule

## Assertion output:

# Output:

Control signal = 2’b00 (count up by 1)

Load value = 4’b0000

Figure 1

Figure 3

Figure 4

Control signal = 2’b00 (count up by 1)

Load value = 4’b0001

Figure 5

Figure 6

Figure 7

Control signal = 2’b00 (count up by 1)

Load value = 4’b1111

Figure 8

Control signal = 2’b00 (count up by 1)

Load value = 4’b1111

Figure 9

Control signal = 2’b01 (count up by 2)

Load value = 4’b0000

Figure 10

Control signal = 2’b01 (count up by 2)

Load value = 4’b0000

Figure 11

Figure 12

Control signal = 2’b01 (count up by 2)

Load value = 4’b0001

Figure 13

Figure 14

Figure 15

Control signal = 2’b01 (count up by 2)

Load value = 4’b1111

Figure 16

Figure 17

Control signal = 2’b10 (count down by 1)

Load value = 4’b0000

Figure 18

Figure 19

Figure 20

Control signal = 2’b10 (count down by 1)

Load value = 4’b0001

Figure 21

Figure 22

Control signal = 2’b10 (count down by 1)

Load value = 4’b1111

Figure 23

Control signal = 2’b11 (count down by 2)

Load value = 4’b0000

Figure 24

Figure 25

Figure 26

Control signal = 2’b11 (count down by 2)

Load value = 4’b0001

Figure 27

Control signal = 2’b11 (count down by21)

Load value = 4’b1111

Figure 28